3GPP TSG-WG SA2 Meeting #175-AH-e	S2-2606335

# Online, June 24 – 30, 2026

Source:	Huawei, HiSilicon

Title:	[KI#1.1, Solution Variant 2A2 update] Update solution with SRF Relocation

Document for:	Approval

Agenda item:	20.6.1

Work Item:	FS_6G_ARC

Abstract of the contribution: This paper updates the solution variant 2A2 with SRF and 6G CN NF relocation.

1.	Introduction

In the current solution variant, the SRF is adopted as termination point for NAS routing functionality. When the UE moves out of the registration area, it needs to invoke Mobility Update Registration procedure towards the 6G CN, and during which the SRF might be relocated.

2.	Text proposal

It is proposed to agree the following changes vs. TR 23.801-01:

* * * First Change * * * *

6.1.1.2.2	Description for Solution Variant 2A2 (NAS routing terminated at 6G SRF)

6.1.1.2.2.1	Motivation

6.1.1.2.2.2	6G architecture

The proposed system architecture to support Modular NAS is shown in Figure 6.1.1.2.2.2.1-1. This architecture shows the non-roaming case.

Editor's note:	The architecture for roaming case is FFS.

Figure 6.1.1.2.2.2-1: System architecture to support Modular NAS

NOTE 1:	The System architecture is mainly used for solution principles illustration without covering all 6G CN NFs in the architecture.

NOTE 2:	 AI Agent (e.g. System AI Agent) aspects are covered by Solution Variant #3.

In this reference architecture, 6G MM and signalling routing functionalities are separated into different network entities: 6G MM is kept in the 6G AMF, while signalling routing is separated into a new dedicated NF called SRF (Signalling Routing Function).

A NAS association is established between the 6G UE and each 6G CN NF. There can be multiple NAS associations between a 6G UE and 6G CN NFs in parallel without dependency within them. The 6G CN NF is the NAS termination point.

The 6G RAN performs SRF selection.

Editor's note:	It is FFS how SRF selection is performed.

The SRF provides following functionalities:

-	Termination of 6G RAN CP interface (i.e. 6G N2).

-	UL/DL NAS signalling routing.

-	6G CN NF discovery and selection.

Editor's note:	The functionality split between the 6G AMF and the SRF, and other functionalities performed by the SRF are FFS, e.g. connection management, paging, temporary UE ID allocation (e.g. 6G-GUTI), termination of NAS security endpoint, etc.

The SRF performs UL NAS signalling routing based on routing information provided by the UE and/or available stored routing information. The SRF performs DL NAS signalling routing based on routing information provided by the 6G CN NF and/or available stored routing information.

Editor's note:	It is FFS whether the SRF needs to store the routing information. If the routing information is stored, the SRF performs the context-based routing; otherwise, contextless-based routing is performed.

The SRF performs 6G CN NF discovery by itself (e.g. directly querying the 6G NRF) or via a dedicated discovery function.

Editor's note:	Details of 6G CN NF discovery and selection by the SRF is FFS.

The SRF supports service-based interface towards 6G CN NFs and an interface towards 6G RAN.

Editor's note:	The interface towards 6G RAN needs to coordinate with RAN3.

The primary authentication for UE access is performed at the 6G AMF.

Editor's note:	The primary authentication needs to coordinate with SA3.

Editor's note:	The serving 6G CN NF relocation due to UE mobility is FFS.

6.1.1.2.2.3	6G NAS Protocol Stack

The control plane protocol stack between the UE and 6G CN NFs for Modular NAS is shown in Figure 6.1.1.2.2.3-1.

Figure 6.1.1.2.2.3-1: Control Plane protocol stack between the UE and 6G CN NFs

NOTE:	The Control Plane protocol stack is mainly used for solution principles illustration without covering all 6G CN NFs.

Unlike 5G NAS in which a single NAS association is used for each access to which the UE is connected, the 6G Modular NAS has following characteristics:

-	There can be multiple NAS termination points located in 6G CN. Each 6G CN NF to which the UE is connected can be the NAS termination point.

-	An NAS association is established between the UE and each 6G CN NF. There can be multiple NAS association between a 6G UE and 6G CN NFs in parallel without dependency within them.

-	A NAS routing layer connection is established between the UE and the SRF for UL/DL NAS signalling routing.

-	In reference point representation, The SRF has direct interface with each 6G CN NF.

6.1.1.2.2.4	NAS identifiers

6.1.1.2.2.4.1	NAS identifiers used for UL NAS signalling routing

6.1.1.2.2.4.1.1	NAS identifiers alternative #1: 6G GUTI allocated by 6G CN NF

A 6G GUTI is allocated by each 6G CN NF independently, which includes the 6G CN NF ID and the temporary UE ID.

The 6G GUTI allocated by different 6G CN NFs has different name and structure, e.g. the 6G GUTI allocated by the 6G AMF is called 6G MM GUTI and is structured as <NF type><MCC><MNC><6G AMF Region ID><6G AMF Set ID><6G AMF Pointer><6G-MM-TMSI>, the 6G GUTI allocated by the 6G SMF is called 6G SM GUTI and is structured as <NF type><MCC><MNC><6G SMF Region ID><6G SMF Set ID><6G SMF Pointer><6G-SM-TMSI>.

The 6G GUTI, as a routing information, is included by the 6G UE in the UL NAS signalling messages and is used by the SRF to route the UL NAS signalling routing to the responding 6G CN NF.

6.1.1.2.2.4.1.2	NAS identifiers alternative #2: 6G GUTI allocated by SRF

A 6G GUTI is allocated by the SRF, which includes the SRF ID and the temporary UE ID, e.g. the 6G GUTI is structured as <MCC><MNC><SRF Region ID>< SRF Set ID>< SRF Pointer><6G-TMSI>. The 6G GUTI allocated by the SRF can be called 6G SRF GUTI and is shared by the UE and all 6G CN NFs.

The 6G SRF GUTI, as a routing information, is included by the 6G UE in the UL NAS signalling messages and is used by the SRF to route the UL NAS signalling routing to the 6G CN NF. The 6G SRF GUTI can be also used by the 6G RAN for SRF selection.

Editor's note:	Whether other routing information is required by the SRF for UL NAS signalling routing is FFS.

Editor's note:	The identifiers needed to route the UL NAS signalling to the corresponding 6G CN NF is FFS.

6.1.1.2.2.4.1.3	NAS identifiers alternative #3: Dedicated Routing ID rather than 6G GUTI

A dedicated Routing ID is allocated by the UE or the 6G CN NF for UL NAS signalling routing.

The dedicated Routing ID, as a routing information, is included by the 6G UE in the UL NAS signalling messages and is used by the SRF to route the UL NAS signalling routing to the 6G CN NF. The dedicated Routing ID can be also used to identify each NAS association between the UE and the 6G CN NF.

6.1.1.2.2.4.2	NAS identifiers used for DL NAS signalling routing

Editor’s note: FFS.

6.1.1.2.2.5	NAS signalling routing

6.1.1.2.2.5.1	NAS signalling routing alternative #1: Contextless-based routing

This NAS signalling routing alternative does not require the SRF to store any routing information.

The UE includes the required routing information in each UL NAS signalling message to the SRF and the SRF directly uses the received routing information for UL NAS signalling routing to 6G CN NF.

The 6G CN NF includes the required routing information in each DL NAS signalling message to the SRF and the SRF directly uses the received routing information for DL NAS signalling routing to the UE.

Editor's note:	The specific required routing information for UL/DL NAS signalling routing includes at least one of, e.g. message type, container type, signalling type, protocol type, operator service type, 6G GUTI, Routing ID, etc., and needs to be decided later.

6.1.1.2.2.5.2	NAS signalling routing alternative #2: Context-based routing

This NAS signalling routing alternative requires the SRF to store required routing information (e.g. as UE routing context). The SRF can store the required routing information locally or in a dedicated 6G CN NF.

The UE includes the required identifier information in each UL NAS signalling message to the SRF, the SRF uses the received identifier information to derive the stored routing information for UL NAS signalling routing to 6G CN NF.

The 6G CN NF includes the required identifier information in each DL NAS signalling message to the SRF, the SRF uses the received identifier information to derive the stored routing information for DL NAS signalling routing to the UE.

Editor's note:	The specific stored routing information for UL/DL NAS signalling routing includes at least one of, e.g. message type, container type, signalling type, protocol type, operator service type, 6G GUTI, Routing ID, etc., and needs to be decided later.

Editor's note:	The specific required identifier information to derive the stored routing information is FFS.

6.1.1.2.2.6	6G CN NF discovery and selection

6.1.1.2.2.6.1	6G CN NF discovery and selection alternative #1: performed by SRF

The SRF directly performs 6G CN NF discovery and selection, e.g. by querying the 6G NRF or based on local configuration.

The SRF can take routing information provided by the UE, and/or available stored routing information into account to select a 6G CN NF to serve the UE.

NOTE:	In this alternative, a default 6G CN NF, if available, can be selected by the SRF.

Editor's note:	Details of 6G CN NF discovery and selection performed by the SRF is FFS.

6.1.1.2.2.6.2	6G CN NF discovery and selection alternative #2: performed by a dedicated NF

The SRF invokes NF discovery and selection service provided by a dedicated NF to select the appropriate 6G CN NF to serve the UE.

Editor's note:	Details of 6G CN NF discovery and selection performed by the dedicated NF is FFS.

6.1.1.2.2.7	NAS security

Editor's note:	Whether a single NAS security termination or NAS security termination per NAS association is FFS and needs to coordinate with SA3.

Editor's note:	How an NAS signalling is security protected and verified at the network side depends on where the NAS security termination is located and is FFS.

6.1.1.2.2.7.1	NAS security alternative #1: Single NAS security termination (B1)

There is a single NAS security termination point located at the SRF and a single NAS security context stored in both the UE and the SRF. The SRF performs the NAS security protection (including integrity protection, ciphering/deciphering) of NAS signalling at the network side. All NAS associations between the UE and 6G CN NFs share the single NAS security context between the UE and the SRF.

6.1.1.2.2.7.2	NAS security alternative #2: Multiple NAS security termination per NAS association (B2)

The NAS security termination point is located at each 6G CN NF. The NAS security context is stored in both the UE and the 6G CN NF per NAS association. The 6G CN NF performs the NAS security protection (including integrity protection, ciphering/deciphering) of associated NAS signalling at the network side.

6.1.1.2.2.8	Procedures

6.1.1.2.2.8.1	Registration procedure

Figure 6.1.1.2.2.8.1-1 intends to indicate an end-to-end call flows for Registration procedure.

Figure 6.1.1.2.2.8.1-1: Registration procedure

0.	The UE in idle mode initiates a registration procedure. The registration procedure can be initial registration or mobility update registration.

1.	The UE sends a UL NAS Transport message carried in a 6G AN message to the 6G RAN. In UL NAS Transport message, the UE includes a NAS message container, i.e. 6G N1 MM container (Registration Request) and required UL routing information as specified in clause 6.1.1.2.2.5.

In Registration Request, the UE can include other required NAS IEs like 6G SUPI, registration type, UE MM capabilities, UE security capabilities, etc.

Editor's note:	NAS IEs included in the Registration Request are FFS.

2.	The 6G RAN determines the target old SRF identified by the UL routing information, if applicable. Otherwise, 6G RAN performs SRF selection to select a new suitable SRF for the UE.

Editor's note:	Details of SRF selection by 6G RAN are FFS and needs to coordinate with RAN.

6G RAN forwards the UL NAS Transport message and the UL routing information to the selected SRF.

3.	If a New SRF is selected, and the SRF is context-based, it triggers the UE Routing Context Retrieval procedure towards the old SRF identified by the received UL routing information.

Editor’s note: procedure for contextless-SRF during registration procedure is FFS.

4.	Based on the received UE routing context, if the SRF determines that a new 6G CN NF (e.g., 6G AMF) needs to be selected, it 	performs 6G CN NF (e.g. 6G AMF) discovery and selection as specified in clause 6.1.1.2.2.6. The SRF transparently forwards the N1 container (Registration Request) to the selected 6G CN NF (e.g. 6G AMF), together with the old 6G CN NF (e.g. 6G AMF) ID if a new 6G CN NF (e.g., 6G AMF) is selected.

5.	The new 6G CN NF (e.g. 6G AMF) determines the old 6G CN NF (e.g. 6G AMF) based on the received old 6G CN NF (e.g. 6G AMF) ID from the SRF.

The new 6G CN NF (e.g. 6G AMF) triggers the UE MM Context Retrieval procedure towards the old 6G CN NF (e.g. 6G AMF).

6. 	The new 6G CN NF (e.g. 6G AMF) performs primary authentication for the UE with involvement of 6G AUSF and 6G UDM.

Editor's note:	Details of primary authentication procedure are FFS and needs to coordinate with SA3.

7.	The 6G CN NF (e.g. 6G AMF) performs NF registration to the 6G UDM followed by MM subscription data retrieval with 6G UDM.

Editor's note:	Details of NF registration and MM subscription data retrieval with 6G UDM are FFS.

8.	The 6G CN NF (e.g. 6G AMF) performs required MM policy association establishment/modification with 6G PCF.

Editor's note:	Details of MM policy association establishment/modification with 6G PCF are FFS.

9.	The 6G CN NF (e.g. 6G AMF) performs UE registration and allocates a routing ID for the UE, as specified in clause 6.1.1.2.2.4, which can be either in the form of 6G MM GUTI or MM Routing ID. The 6G MM GUTI contains NF type, 6G CN NF (e.g. 6G AMF) ID and also the temporary UE ID. If the UE registration can be accepted, the 6G CN NF (e.g. 6G AMF) sends 6G N1 MM container (Registration Accept) together with DL routing information to the SRF. In Registration Accept, the 6G CN NF (e.g. 6G AMF) also includes the allocated routing ID sent to the UE.

Editor's note:	NAS IEs included in the Registration Accept are FFS.

10.	The SRF establishes UE-CN NF binding/mapping information between the UE and the serving 6G CN NF (e.g. 6G AMF) using the received UL/DL routing information. The SRF will use such UE-CN NF binding/mapping information for subsequent UL/DL 6G MM message routing during the ongoing 6G MM procedure, i.e. registration here.

The SRF then sends a DL NAS Transport message (N1 container (Registration Accept)) to the UE during which the N1 container was transparently forwarded. The new SRF allocates a new SRF GUTI and provides it to the UE within the DL NAS Transport message.

Upon receipt of Registration Accept message, the UE moves to the connected mode.

11.	If a new NAS identifier as specified in clause 6.1.1.2.2.4 is allocated, the UE needs to ack it by sending an Acknowledge message to the SRF, which can be piggybacked with 6G N1 MM Container (Registration Complete) for the 6G CN NF (e.g. 6G AMF).

12.		The SRF uses UE-CN NF binding/mapping information to route 6G N1 MM container (Registration Complete) to the current serving 6G CN NF (e.g. 6G AMF). As here the UE was already in the connected mode and within the same ongoing MM procedure, the UE needs not to provide the UL routing information.

After completion of registration procedure, if there is no need to keep the UE in the connected mode, the 6G AMF indicates the SRF to release the established routing layer connection to move the UE to the idle mode.

6.1.1.2.2.8.2	PDU session establishment procedure

6.1.1.2.2.8.2.1	PDU session establishment alternative #1: without 6G AMF involvement

Figure 6.1.1.2.2.8.2.1-1 intends to indicate an end-to-end call flows for PDU session establishment procedure.

Editor's note:	In this alternative, whether more than one 6G CN NFs (e.g. 6G SMF) are involved to realize 6G SM functionalities is FFS.

Figure 6.1.1.2.2.8.2.1-1: PDU session establishment procedure

0.	The UE is in registered state, in idle mode and has pending UL user data to be sent. There is no appropriate PDU session established for such UL user data transport and hence the UE decides to initiate a new PDU session establishment procedure.

1.	The UE sends a UL NAS Transport message carried in a 6G AN message to the 6G RAN. In UL NAS Transport message, the UE includes a NAS message container, i.e. N1 container (PDU Session Establishment Request (DNN, PDU session ID)) and required UL routing information as specified in clause 6.1.1.2.2.5.

In PDU Session Establishment Request, the UE can include other required NAS IEs like PDU session type, UE SM capabilities, etc.

Editor's note:	NAS IEs included in the PDU Session Establishment Request are FFS.

2.	6G RAN performs SRF selection to select a suitable SRF for the UE. 6G RAN can select an SRF based on UL routing information or local configuration.

Editor's note:	Details of SRF selection by 6G RAN are FFS and needs to coordinate with RAN.

6G RAN forwards the UL NAS Transport message and the UL routing information to the selected SRF.

Upon receipt of UL NAS Transport message, a routing layer connection between the UE and the SRF is established, i.e. the UE moves to the connected mode.

3.	The SRF performs 6G CN NF (e.g. 6G SMF) discovery and selection as specified in clause 6.1.1.2.2.6. The SRF transparently forwards the N1 container (PDU Session Establishment Request) to the selected 6G CN NF (e.g. 6G SMF).

4.	The 6G CN NF (e.g. 6G SMF) performs SM subscription data retrieval with 6G UDM.

Editor's note:	Details of SM subscription data retrieval with 6G UDM are FFS.

5.	The 6G CN NF (e.g. 6G SMF) performs required SM policy association establishment/modification with 6G PCF.

Editor's note:	Details of SM policy association establishment/modification with 6G PCF are FFS.

6.	The 6G CN NF (e.g. 6G SMF) performs 6G UPF selection to select a suitable 6G UPF to serve the UE and then initiates a 6G N4 Session Establishment procedure to the selected 6G UPF to configure required parameters and session contexts for the established PDU session. The 6G UPF provides its CN Tunnel Info to the 6G CN NF (e.g. 6G SMF) for UL user data transport.

7.	The 6G CN NF (e.g. 6G SMF) performs 6G PDU session establishment and allocates a 6G SM GUTI for the UE. The 6G SM GUTI contains 6G CN NF (e.g. 6G SMF) ID and also the temporary UE ID. If the PDU session establishment can be accepted, the 6G CN NF (e.g. 6G SMF) sends N1 container (PDU Session Establishment Accept (6G SM GUTI, PDU session ID)), N2 container (PDU session ID, CN Tunnel Info) and the DL routing information to the SRF.

Editor's note:	Details of 6G CN NF (e.g. 6G SMF) performs PDU session establishment are FFS.

Editor's note:	IEs included in the PDU Session Establishment Accept and N2 container are FFS.

8.	The SRF establishes UE-CN NF binding/mapping information between the UE and the serving 6G CN NF (e.g. 6G SMF) using the received UL/DL routing information. The SRF will use such UE-CN NF binding/mapping information for subsequent UL/DL 6G SM message routing during the ongoing 6G SM procedure, i.e. PDU session establishment here. The SRF maintains the UE-CN NF binding/mapping information until the UE enters the idle mode.

9.	The SRF then sends a 6G N2 PDU Session Request (N2 container (PDU session ID, CN Tunnel Info)) to the 6G RAN to establish required user plane resources for the established PDU session. The 6G RAN then performs RRC Connection Reconfiguration procedure to the UE over 6G radio.

10.	The SRF also sends a DL NAS Transport message (N1 container (PDU Session Establishment Accept)) to the UE during which the N1 container was transparently forwarded.

Once the PDU session and its user plane resources are established, the UE can send the pending UL user data to the 6G RAN and then to the 6G UPF.

11.	After the user plane resources are established, the 6G RAN sends a 6G N2 PDU Session Response (N2 container (PDU session ID, AN Tunnel Info)) to the SRF. The SRF then forwards the N2 container to the current serving 6G CN NF (e.g. 6G SMF) using UE-CN NF binding/mapping information. Here the 6G RAN provides its AN Tunnel Info for DL user data transport.

12.	The 6G CN NF (e.g. 6G SMF) initiates a 6G N4 Session Modification procedure to the serving 6G UPF to provide 6G RAN AN Tunnel Info for DL user data transport.

After this step, the 6G UPF can send the DL user data to the 6G RAN and then to the UE.

6.1.1.2.2.8.2.2	PDU session establishment alternative #2: with 6G AMF involvement

Figure 6.1.1.2.2.8.2.2-1 intends to indicate an end-to-end call flows for PDU session establishment procedure.

Figure 6.1.1.2.2.8.2.2-1: PDU session establishment procedure

0.	6G-AMF obtains UE subscription data during registration procedure.

1.	Same as step 1 in clause 4.3.2.2.1 of TS 23.502 [3].

2.	After receiving the PDU session establishment request message from 6G-UE including the Message Routing ID and corresponding PDU session parameters, SRF sends the parameters (e.g. UE requested DNN, PDU session id) to 6G-AMF for 6G-SMF selection. 6G-AMF performs 6G-SMF selection with considering the parameters from SRF, and UE's subscription data. 6G-AMF may query 6G-NRF for 6G-SMF selection.

3.	6G-AMF sends the selected 6G-SMF info, replaced DNN to SRF.

4.	SRF forwards the PDU session establishment request, replaced DNN to 6G-SMF, and stores the mapping between the Message Routing ID and 6G-SMF info for the subsequent SM message routing. SRF may send 6G-AMF info to 6G-SMF for downlink data notification.

5.	Same as step 4 – 10 in clause 4.3.2.2.1 of TS 23.502 [3] by replacing AMF with SRF.

6-8.	PDU session establishment accept message is provided to 6G-UE and the radio resource setup is performed correspondingly. In case the PDU Session Secondary Authentication/Authorization applies, the DN Request Container information is transferred between SMF and SRF directly.

9.	6G2 PDU Session Response from 6G-RAN to SRF.

10.	SRF syncs up the PDU session status with 6G-AMF.

11.	Same as step 15 – 21 in clause 4.3.2.2.1 of TS 23.502 [3].

6.1.1.2.2.9	Services, Entities and Interfaces

6.1.1.2.2.10	Issues

* * * End of Changes * * * *